# Introduction

AeroMind is a hybrid autonomous drone platform developed by HackSmiths. It is built for the following.

- Operational use in inspection, mapping, agriculture, surveillance, and emergency response

- Unified operator control, live telemetry, camera and map overlay, and AI-based identification

- Administered access with device binding, approval, and forced control

- Four layers: Flutter app, FastAPI backend, Raspberry Pi 4, and F7 / INAV

NOTE The operator never commands the flight controller directly. This report covers the aim, the layered architecture, the operating flow, and the implementation of each layer.

# Aim and Objectives

The aim of this project is to design and implement AeroMind, a hybrid autonomous drone system that combines real-time operator control, live telemetry, visual overlay, AI-based multi-model identification, and secure administered access within one layered platform.

The specific objectives of the project are as follows.

- Design a Flutter mobile application that provides dual-joystick flight command, live telemetry, and a camera or map overlay.

- Implement a FastAPI backend for authentication, device binding, session control, telemetry exchange, and AI service delivery.

- Deploy a Raspberry Pi 4 companion computer as the onboard bridge between backend services and the flight controller.

- Configure an INAV-based F7 flight controller to retain low-level attitude control and stable flight behavior.

- Support AI-based multi-model identification on visual data and return results to the operator during a session.

- Enforce secure operation through device binding, administrator approval, and forced control.

The work is limited to the items listed above.

# Architecture

AeroMind is four layers on one path. Each layer talks only to its neighbor. The operator never sends motor or MSP commands to the flight controller.

| Flutter Application | FastAPI Backend | Raspberry Pi 4 | F7 / INAV |
| --- | --- | --- | --- |
| Operator client | System authority | Onboard bridge | Real-time flight stack |
| Joysticks, overlays, SQLite | JWT, bind, one controller | MSP, sensors, 4G | Modes, PID, motors, ESCs |
| Commands out | WebSocket to the aircraft | MSP setpoints to INAV | Attitude and throttle |
| Camera / map / status in | Redis, WebRTC, AI results | Telemetry and video up | IMU, barometer, current |

Figure 1. Layered architecture of AeroMind. Commands travel left to right. Telemetry, video, and identification results travel right to left.

## Flutter Mobile Application

The Flutter application is the operator client.

- Captures dual-joystick and button input

- Displays live telemetry

- Presents camera or map as an overlay

- Stores login and settings in SQLite

- Sends every command to the FastAPI backend

## FastAPI Backend

The FastAPI backend is the system authority.

- Route groups: Auth, Users, Admin, Connection, Commands, Telemetry, and AI

- JWT authentication with device binding

- Only one active controller at a time

- Latest telemetry stored in Redis

- Commands to the companion computer over WebSocket

- Live camera and audio signaling over WebRTC

- Multi-model, multi-level identification

- Administrator approval, access policy, and forced control

NOTE External language models (OpenAI, Groq, Gemini, and Grok) only refine identification results inside the service. They are not the operator interface.

## Raspberry Pi 4 Companion Computer

The Raspberry Pi 4 is the onboard bridge.

- Modules: Core, Sensors, Flight, Autonomy, Network, and Utils

- Core: configuration, state management, and logging

- Sensors: rain, LIDAR, ultrasonic, battery, environment, GPS, IMU, camera, and microphone

- Flight: MSP communications, flight-controller interface, and fail-safe

- Autonomy: decision engine, state machine, and safety rules

- Network: telemetry sender, command listener, and SIM7600 4G handler

- Utils: helpers and retry / failure logic

## F7 Flight Controller

The F7 flight controller, running INAV, is the real-time flight stack.

- UART MSP to the Raspberry Pi and UART for the receiver

- Sensors: IMU (gyro and accelerometer), barometer, and current

- Flight modes: stabilize / angle, altitude hold, and multi-stage fail-safe

- Motors and ESCs: mapping, DShot protocol, and motor direction

- PID tuning with gyro and D-term filters

- MSP enable, baud rate, and feature flags

NOTE Inner-loop stability stays on INAV. Policy, media, and identification stay in software that can be updated independently.

# How the System Works

An operator action travels the same path every time. Nothing on the handset talks to the flight controller.

Command path

| 01 | 02 | 03 | 04 | 05 |
| --- | --- | --- | --- | --- |
| Operator | Flutter App | FastAPI | Raspberry Pi | F7 / INAV |
| Input | Command | Auth and policy | MSP bridge | Flight |

Return path

| 01 | 02 | 03 | 04 |
| --- | --- | --- | --- |
| F7 / INAV | Raspberry Pi | FastAPI | Flutter App |
| Sensors | Video and telem. | Redis · WebRTC · AI | Overlay |

Figure 2. End-to-end operating flow of AeroMind.

The operating sequence is as follows.

- The operator signs in. An unapproved account is blocked.

- The operator pairs a bound aircraft and opens the controller.

- Stick and button inputs go to the FastAPI backend.

- The backend checks the JWT, the device binding, and that this session is the only active controller.

- The command is sent to the Raspberry Pi over WebSocket.

- The Pi forwards a validated setpoint to INAV over MSP.

- INAV moves the airframe and keeps attitude stable.

- Telemetry returns Pi → Redis → application.

- Live camera and audio use WebRTC. If the camera is off, the map overlay is shown.

- AI identification can be run on visual data. External language models only refine those results internally.

- An administrator can approve a session or seize control at any time.

NOTE Only one controller may be active. An unapproved session cannot command the aircraft.

# Features

AeroMind is a complete hybrid drone platform. Its overall capabilities are as follows.

- Hybrid operation: the same aircraft supports direct human control and onboard autonomy

- End-to-end remote flight over 4G, not a private radio-only link

- Live telemetry with camera or map overlay

- Photo and video capture from the operator client

- Multi-model, multi-level identification, with optional internal language-model refinement

- Secure access through JWT, device binding, approval, and a single active controller

- Administrator override and forced control

- Onboard fail-safe on the INAV flight stack

- Modular four-layer design so each layer can be replaced without rewriting the others

- Scalable users, devices, and aircraft through the backend

- Extensible AI: new models can be added without changing the flight controller

- Room to grow autonomy on the companion computer while INAV stays the real-time core

- Wide-area operation through SIM7600 4G backhaul

NOTE Software features scale on the backend and companion computer. The flight controller stays dedicated to real-time stability.

# Components & Tools

Each part has a fixed job on this aircraft. Photos sit beside the use of that part. Supporting hardware is listed with a collage.

## Flight Hardware

- DYS FC722 Flight Controller

- A2212/13 1400KV Brushless Motors ×4

- SimonK 30A ESCs ×4

- RadioMaster RP1 ELRS Nano Receiver

- ELRS 2.4 GHz Radio Transmitter

- Caddx Baby Ratel 2 FPV Camera

- HGLRC Zeus 800mW 5.8 GHz VTX

- Foxeer M10Q 5883 GPS + Compass

- Pro-Range 11.1V 5200mAh 40C 3S LiPo Battery

- Matek PDB-XT60

- HCS-150A Hall Current Sensor

- INMP441 Digital MEMS Microphone

- MAX98357A I2S Class-D Amplifier

- INVENTO 0.5W 8Ω Speaker

- DYP-A02YYUW Waterproof Ultrasonic Sensor

- Contact-Type Rain Sensor

- Q450 V3 Quadcopter Frame

- 1045 (10×4.5") CW/CCW Propeller Set

- RX5808 Dual Diversity 5.8 GHz Video Receiver

- USB HDMI Capture Card